WALSCHAERT OUTSIDE ADMISSION SPECIAL INSTRUCTIONS

The program starts with the common arrangement of the reverse arm with its lifting arm pointing forward and the radius rod down for forward.

If the lifting arm is pointing to the rear, make the lifting arm negative.

If the radius rod is up for forward then make the eccentric circle negative to place the eccentric crank so it leads the main crank.

If the reverse arm and the lifting arm are in front of the link, then make the radius rod extension negative.

If the radius rod extension slides in a bushing at the end of the lifting arm with no lifting link, make the lifting link and the radius rod extension zero. The lifting link will rubberband causing a calculation error beep. Turn off the calculation error beep.

Remember to recalculate the reverse arm center position if dimensions arround the link and the reverse lever have been changed and the auto calculation was not activated.