STEPHENSON OUTSIDE ADMISSION SPECIAL INSTRUCTIONS

The program starts with the common arrangement of the reverse arm with its lifting arm pointing forward and using a rocker for indirect motion to the valve. The expansion link is down for forward.

If the lifting arm is pointing to the rear, make the lifting arm negative.

If the valve has direct motion, make the lower part of the rocker large (20X to 50X), make the upper part of the rocker equal to the negative of the lower part of the rocker, make the vertical position of the rocker center pivot equal to the lower part of the rocker, make the backset of the upper part of the rocker equal to zero, make the distance between the cylinder center line and the valve center line equal to zero, rotate both eccentrics to correct the timing. The valve rod should not be zero, shorten the valve stem so the sum of the two is equal the true valve stem length.

Remember to recalculate the reverse arm center position if dimensions arround the link and the reverse lever have been changed and the auto calculation was not activated.

ADJUSTING STEPHENSON VALVE GEAR

For this to work properly, both eccentrics, valve rods and valve stem must be adjustable.

The engine is on some rollers or rail. Tram marks or points are required to show 0 deg crank position for TOP DEAD CENTER CRANK FORWARD and 180 deg crank position for TOP DEAD CENTER CRANK BACK.

Remove steam chest covers. Adjustments should be tight.

To check approximate eccentric position, set gear to forward, roll engine forward watching valve. When passing TDC FORWARD the front port should just open and passing TDC BACK the rear port should just open. If this is wrong, rotate forward eccentric to correct. Set gear in reverse, roll engine back, front port should just open when passing TDC FORWARD and rear port should just open when passing TDC BACK. Correct if necessary.

The forward eccentric rod should be adjusted to its design length and tightened.

Set gear to forward. Roll engine forward to TDC FORWARD. Note valve position on front port. Roll engine forward to TDC BACK. Note valve position on rear port. Adjust valve stem for equal valve positions on both ports.

Set gear forward. Roll engine forward to TDC FORWARD. Adjust forward eccentric so front port is just ready to open.

Set Gear to reverse. Roll engine backwards to TDC FORWARD. Note valve position on front port. Roll engine backwards to TDC BACK. Note valve position on rear port. Adjust reverse eccentric rod for equal valve positions on both ports.

Set gear in reverse. Roll engine backwards to TDC FORWARD. Adjust reverse eccentric so front port is just ready to open.

Repeat adjustments until front port and rear port are just ready to open at TDC for both forward and reverse.

Set gear to mid position. Roll engine forward to TDC FORWARD. Valve position on front port should be open by LEAD. About 0.020 to 0.025 inches on 1.5 inch scale. Roll engine to TDC BACK. Lead should be the same as at TDC FORWARD. If not recheck adjustments.

If lead is too large, reduce the length of the upper rocker, or increase the length of the lower rocker or reduce the throw of the eccentric. Percent change of rocker is close to percent change of lead. Then readjust again as above. Eccentric angles will also change.