HELMHOLTZ  OUTSIDE ADMISSION SPECIAL INSTRUCTIONS

Helmholtz has two setups for outside admission. The second setup has the

radius rod above the link block. To setup this setup set the Eccent Crank

Circle Dia negative, the Lift Arm negative and the Lift Link Lower negative.

An example is data file Helm.dhz