SKINNER SPECIAL INSTRUCTIONS

Patent 1,927,524   Sept. 19, 1933

Adjusting the valves

Set the cutoff slide bar to Up or maximum cutoff.

Select a Cam Ramp Angle and adjust it.

Stop motion by setting the Left Crank Angle to 0.0 degrees.

Adjust the Left Admission Cam Calibration Angle so the left upper valve is at its opening point.

Set the Left Crank Angle to 180.0 degrees.

Adjust the Cam Maximum Radius Arc Angle so the left lower valve is at its opening point.

Readjust the Left Admission Cam Calibration Angel for the desired lead.

Adjust the Left Cutoff Cam Calibration Angel for maximum cutoff or 80 percent.

Set the cutoff slide bar to Dn or minimum cutoff.

Adjust the Governor Controlled Cutoff Cam Angle to the desired minimum cutoff.

The Governor Controlled Cutoff Cam Angle cannot be changed dynamically.

Adjust the Right Admission and Right Cutoff Cams Calibration Angles to 90.0 degrees above the Left Cams Calibration Angles.

Use small steps for moving the crank.

Zoom in to see detail.